Table S2. SRM acquisition parameters and instrumental detection limits (IDL) for target compounds, inner standard and surrogates by GC-MS/MS

| Compound | Abbreviation | RT min | Quantitative transition (Q) | Qualitative transition (q) | Q/q CE (eV) | IDL (μg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| acenaphthene | Acp | 10.37 | 153.3→152.2 | 153.3→126.7 | 20/30 | 0.07 |
| acenaphthylene | Acpy | 10.05 | 152.1→150.2 | 152.1→126.1 | 30/25 | 0.10 |
| anthracene | Ant | 14.68 | 178.3→176.2 | 178.3→151.1 | 30/35 | 0.46 |
| fluorene | Flu | 11.48 | 166.0→165.0 | 166.0→115.3 | 20/40 | 0.04 |
| fluoranthene | FluA | 20.66 | 202.1→200.2 | 202.1→151.9 | 35/35 | 0.24 |
| phenanthrene | Phe | 14.48 | 178.2→176.2 | 178.2→151.0 | 30/35 | 0.47 |
| pyrlene | Pyr | 22.17 | 228.3→226.1 | 202.1→151.7 | 40/40 | 0.23 |